Oil Production Prediction & Forecasting Using Machine Learning.

--Final Project Overview

This project focuses on building a reliable machine learning system to predict and forecast oil production using real-world operational data. The objective was not just accuracy, but developing a trustworthy and industry-relevant model by addressing common pitfalls such as data leakage and improper time series handling.

--Problem Statement

Oil production is influenced by multiple operational factors such as pressure, temperature, and choke settings. Accurately predicting production enables:

- Better production planning

- Early detection of performance decline

- Data-driven operational decisions

--Data Understanding & Preprocessing

The dataset contains daily production records from multiple wells, including:

- Downhole and wellhead pressure

- Temperature measurements

- Annulus pressure

- Choke size

- Oil production volume

--Key Data Processing Steps

- Converted date columns to datetime format

- Handled missing values using forward filling

- Aggregated production data by date to create a consistent time series

- Removed zero/invalid production records

-- Critical Insight: Avoiding Data Leakage

Initial experiments showed near-perfect performance (R² ≈ 0.998).

However, this was identified as data leakage, caused by including highly correlated variables such as gas and water production.

-- Fix:

- Removed leakage-prone features

- Retained only independent operational variables

This ensured the model learns real physical relationships, not shortcuts.

🧠 Model Development

A regression model was built using XGBoost to capture non-linear relationships between operational parameters and oil production.

📊 Model Performance

| Metric | Value |
| --- | --- |
| R² Score | 0.97 |
| MAE | 99 |
| RMSE | 216 |

-- Interpretation

- The model explains 97% of the variance in oil production

- Prediction error is approximately ~2% of production scale

- Performance is both accurate and realistic

📈 Model Validation

To ensure robustness:

- Used proper train-test evaluation

- Conducted residual analysis

- Verified that residuals are:

  - Centered around zero

  - Approximately normally distributed

  - Mostly random

--This confirms the model is unbiased and reliable

🔍 Key Insights

- Choke size and pressure variables significantly influence oil production

- Oil production shows a natural decline trend over time

- The model performs well across most production ranges

- Occasional large errors indicate operational anomalies or sudden production changes

🔮 Time Series Forecasting

Time series forecasting was implemented using Prophet.

⚠️ Important Learning

- Long-term forecasting produced unrealistic results (e.g., negative production)

- This highlighted the importance of forecast horizon selection

✅ Final Approach

- Aggregated data into a clean daily time series

- Limited forecasting to short-term horizons

- Ensured predictions remain physically meaningful

💡 Business Impact

This solution can support:

- Short-term oil production forecasting

- Operational decision-making

- Identification of key production drivers

- Monitoring well performance decline

🛠️ Tools & Technologies

- Python (Pandas, NumPy)

- XGBoost

- Prophet

- Matplotlib & Seaborn

-- Final Conclusion

This project demonstrates the importance of building trustworthy machine learning systems, not just high-performing ones. By addressing data leakage, validating model behavior, and properly handling time series data, the final solution achieves both high accuracy and real-world reliability.